| Project / BMP # |  | Date |  | Initials |
| --- | --- | --- | --- | --- |

VA DEQ STORMWATER DESIGN SPEC. NO. 15:  EXTENDED DETENTION POND

Level 1			Level 2

MINIMUM DESIGN CRITERIA:

- Level 1 Extended Detention Pond designs must meet the following criteria. Spec No. 15 Sec 3

The treatment volume of the swale must be calculated using the following equation:

Tv = [(1.0)(Rv)(A)]/12 – the volume reduced by an upstream SWM facility.

There must be at least 15% of the treatment volume in the permanent pool (includes forebay and micropool).

The overall basin length to width must be 2:1 or larger.

The ratio of the shortest flow path (closest inlet to the outlet) to the overall length must be 0.4 or more.

The average Tv extended detention time cannot exceed 24 hours.

The vertical Tv extended detention cannot exceed 5 feet.

A forebay and micro-pool must be incorporated into the pond design.

The CDA must be less than 10 acres.

- Level 2 Extended Detention Pond designs must meet the following criteria. Spec No. 15 Sec 3

The treatment volume of the swale must be calculated using the following equation:

Tv =  [(1.25)(Rv)(A)]/12 – the volume reduced by an upstream SWM facility.

There must be at least 40% of the treatment volume in the permanent pool (includes forebay, micropool, deep pool, and wetland).

The overall basin length to width must be 3:1 or larger.

The ratio of the shortest flow path (closest inlet to the outlet) to the overall length must be 0.7 or more.

The average Tv extended detention time cannot exceed 36 hours.

The vertical Tv extended detention cannot exceed 4 feet.

Trees and wetlands must be included in the planting plan.

Additional cells or features (deep pools, wetlands, etc.) must be included in the design plan.

The CDA must be at least 10 acres.

PLAN REQUIREMENTS:

- Provide a site map identifying pertinent information regarding the Extended Detention (ED) Pond.

The contributing drainage area (CDA) boundaries, acreage, and land cover;

Topography of the site area including the ED pond area, its CDA, and any pre-treatment practices;

There must be at least 10 feet from property lines.

There must be at least 25 feet from buildings.

There must be at least 100 feet from septic fields and/or private wells.

There must be at least 100 feet between any well and the permanent pool elevation.

The locations of all conveyance system outfalls into basin must be shown.

Inlet pipe inverts must be designed to be located at or slightly below the forebay pool elevation.

The location and dimensions of the riser and barrel must be shown.

All cells or features (deep pools, wetlands, etc.) must be identified.

- Provide profiles and section views that show the following:

| Top of dam | Emergency spillway elevation |
| --- | --- |
| Side slopes | Principal spillway crest |
| 10-year storm elevation | 100-year storm elevation |
| Tv (detention) volume elevation | Volumes of various cells |
| Energy balance volume detention elevation | Internal baffles including material and elevation information |
| Safety and aquatic benches | Principal spillway and embankment |
| Internal baffles including material and elevation information | Basin bottom and water surface elevations for permanent pool(s), including forebays, micropools, deep pools, and/or wetlands. |

- Provide a plan view showing:

| Overall basin grading | Forebays | Micropools |
| --- | --- | --- |
| Deep pools | Wetland cells | Safety and aquatic benches |
| Internal baffles/barriers | Shortest flow path | Discharge points into the basin |
| Emergency spillway and exit channel | Sizing and dimensions for all components | Delineation of the 50-year (if applicable) and 100-year storm elevation in and around the basin |

- Provide a detail of the principal outlet structure showing:

| Embankment elevations | Riser crest elevation | Trash racks |
| --- | --- | --- |
| Water quality orifice sizing and elevations | Dewatering device anti-clogging information | Material and sizing information for all components |
| Tv, 1-year, 10-year, 50-year, and 100-year storm elevations, where applicable |  |  |

- If the forebay discharges to a pool, the elevation of the stabilized overflow spillway component of the forebay must be set at the permanent pool elevation.

- If the forebay discharges to a pool, the elevation of the stabilized overflow spillway component of the forebay must be designed to allow flow to exit the forebay at non-erosive velocities during the 2-year design storm.

- The maximum allowable longitudinal slope through the pond is 1%.

- Inlet pipe inverts must be located at or slightly above the stabilized overflow spillway component of the forebay.

COMPUTATIONS:

- Hydrologic analysis must be based on a 24-hour storm event using site specific rainfall precipitation frequency data recommended by the U.S. National Oceanic and Atmospheric Administration (NOAA) Atlas 14.

- The U.S. Department of Agriculture’s Natural Resources Conservation Service (NRCS) synthetic 24-hour rainfall distribution and models, including but not limited to TR-55 and TR-20; hydrologic and hydraulic methods developed by the U.S. Army Corps of Engineers;  or other standard hydrologic and hydraulic methods.

- The Rational Method may only be used for evaluating peak discharges

- The Modified Rational Method may be use for evaluating volumetric flows to stormwater conveyances with a drainage areas of 200 acres or less.

- Provide pre- and post-development hydrologic, hydraulic, and routing information for the 1- year, 2- year, 10- year, and 100-year, 24-hour storms, as needed, for all extended detention basins.:

- Provide pre- and post-development hydrologic, hydraulic, and routing information for the 1- year, 2- year, 10- year, 50-year, and 100-year, 24-hour storms, as needed, for 50/10 detention facilities.

- Provide the complete input used for the hydrograph generation on the plans

- Provide calculations to show how the required treatment volume (Tv) is determined.

LANDSCAPING:

- Provide a landscape plan that includes the following:

Delineation of pond-scaping zones within both the pond and buffer;

The selection of corresponding plant species;

A planting plan;

The sequence for preparing the wetland bed, if one is incorporated;

Sources of native plant material;

Woody vegetation may not be planted or allowed to grow within 15 feet of the toe of the embankment, nor within 25 feet of the principal spillway structure.

- Landscape plants must be selected from appropriate tables contained in Appendix E:  Landscaping located in the Virginia BMP Clearinghouse (http://vwrrc.vt.edu/swc/NonProprietaryBMPs.html).

- Trees, shrubs or any other woody vegetation are not permitted on the embankment or adjacent areas extending at least 25 feet beyond the embankment toe and abutment contacts.

- The construction sequence must specify that a 6” layer of topsoil is required for all areas of the basin that will be stabilized with a permanent stand of vegetation.  The topsoil layer must be clearly shown on all basin related details to ensure that all post-construction elevations reflect the topsoil application.

- Landscaping of the basin slopes that requires mulching, spraying, etc. must be limited to areas above the elevation of the top of the principal outlet structure elevation and must be done in a manner that does not inhibit maintenance access.

MAINTENANCE:

- A Maintenance Agreement is required, identifying the person or organization responsible for maintenance and authorizing access for inspections and maintenance.

- Provide a summary of the long term maintenance requirements for the SWM facility on the SWM plan.

- Inspections required by the Maintenance Agreement must be conducted by 1) a person who is licensed as a professional engineer, architect, landscape architect, or land surveyor pursuant to Sec. 54.1-400 et seq. of the Code of Virginia; 2) a person who works under the direction and oversight of a licensed professional engineer, architect, landscape architect, or land surveyor; or 3) a person who holds a Stormwater Inspector or Stormwater Combined Administrator certificate of competence from the State Water Control Board.

CONSTRUCTION:

- The sequence of construction must address the SWM facility installation and appropriate inspections, including:  initial site preparation, excavation/grading, and installation of the embankment, principal outlet structure, and emergency spillway.

- The sequence of construction must clearly state that a construction record drawing and certification that the stormwater management facility has been constructed in accordance with the approved plan must be submitted to the County and approved prior to Bond  release.