| Project / BMP # |  | Date |  | Initials |
| --- | --- | --- | --- | --- |

VA DEQ DESIGN SPEC. NO. 11:  WET SWALE

Level 1			Level 2

MINIMUM DESIGN CRITERIA:

- The Level 1 Wet Swale design must meet the minimum design criteria. Spec No. 11 Sec 3

The treatment volume of the swale must be calculated using the following equation:

(1 inch)(Rv)(A)]/12 – the volume reduced by an upstream RR BMP.

The swale slope must be < 2%.

- The Level 2 Wet Swale design must meet the minimum design criteria. Spec No. 11 Sec 3

The treatment volume of the swale must be calculated using the following equation:

(1.25 inch)(Rv)(A)]/12 – the volume reduced by an upstream RR BMP.

The swale slope must be < 1%.

The facility must be designed off-line.

The planting plan must incorporate wetland species and trees.

PLAN REQUIREMENTS:

- Provide a site map identifying pertinent information regarding the Wet Swale.

The contributing drainage area (CDA) boundaries, acreage, and land cover.

Topography of the site area including the Wet Swale

- Provide a plan view showing:

Overall Wet Swale grading;

The layout and dimensions of the Wet Swale and pre-treatment device(s); as well as benches, check dams, and all conveyance system outfalls into the swale;

Check dam locations;

Adequate maintenance access to the facility is provided.

- Provide profiles, section views, and details that show the following:

A trapezoidal or parabolic cross section.

Side slopes

Channel bottom width

Longitudinal slope

Check dam details

10-year storm elevation

2-year storm elevation

Tv (detention) volume elevation

Energy balance volume detention elevation

Outfall, tailwater conditions, and downstream limit analyses

COMPUTATIONS:

- Provide appropriate hydraulic calculations.

Hydrologic analysis must be based on a 24-hour storm event using site specific rainfall precipitation frequency data recommended by the U.S. National Oceanic and Atmospheric Administration (NOAA) Atlas 14.

The U.S. Department of Agriculture’s Natural Resources Conservation Service (NRCS) synthetic 24-hour rainfall distribution and models, including but not limited to TR-55 and TR-20; hydrologic and hydraulic methods developed by the U.S. Army Corps of Engineers;  or other standard hydrologic and hydraulic methods.

The Rational Method may only be used for evaluating peak discharges.

The Modified Rational Method may be use for evaluating volumetric flows to stormwater conveyances with a drainage areas of 200 acres or less.

Provide pre- and post-development hydrologic and hydraulic information for the 1- year, 2- year, and 10- year, 24-hour storms, as needed.

Provide the calculated channel dimensions designed based on peak flow rate.

Provide verification of the hydraulic capacity using Manning’s Equation or an accepted equivalent method.

Provide the complete input used for the hydrograph generation on the plans.

CHECK DAMS:

- Check dams, if used, meet the following design criteria:  The items checked blow have not been properly addressed:

Check dams are composed of permanent, non-erodible material or should be configured with elevated driveway culverts; creosote treated materials and stone check dams are unacceptable.

Check dams must be firmly anchored into the side slopes and channel bottom.

The maximum check dam height is 12 inches or 18 inches with additional design elements to ensure stability of the check dam for challenging sites.

Armoring is provided, if necessary, at the downstream toe of the check dam.

Check dams are designed with a center weir sized to pass the channel design peak flow (10-year storm event for man-made channels).

The average ponding depth throughout the channel is 12 inches and must not exceed 18 inches.

The ponded water at a downstream check dam does not touch the toe of the upstream check dam.

LANDSCAPING:

- Provide a landscape plan that specifies the following:

The landscaping plan must indicate the methods to be used to establish and promote long term vegetative cover in the SWM facility and its buffer area.

MAINTENANCE:

- A Maintenance Agreement is required, identifying the person or organization responsible for maintenance, authorizing access for inspections and maintenance, and a providing a maintenance inspection checklist.

- Provide a summary of the long term maintenance requirements for the SWM facility on the SWM plan.

- Inspections required by the Maintenance Agreement must be conducted by 1) a person who is licensed as a professional engineer, architect, landscape architect, or land surveyor pursuant to Sec. 54.1-400 et seq. of the Code of Virginia; 2) a person who works under the direction and oversight of a licensed professional engineer, architect, landscape architect, or land surveyor; or 3) a person who holds a Stormwater Inspector or Stormwater Combined Administrator certificate of competence from the State Water Control Board.

CONSTRUCTION:

- The sequence of construction must address the SWM facility installation and appropriate inspections, including:  initial site preparation, excavation/grading, and installation of the embankment, principal outlet structure, and emergency spillway.

- The sequence of construction must clearly state that a construction record drawing and certification that the stormwater management facility has been constructed in accordance with the approved plan must be submitted to the County and approved prior to Bond release.